
Submersible Pumps

Fact Sheet. 

Pumps are used to circulate water around the pond. 

They will supply waterfalls, fountains, UVs, filters etc.. etc..

Submersible pumps are very easy to install, very safe to use and require little maintenance.

Pumps are available in a wide range of sizes, usually  being rated in gallons per hour \ litres per hour. When deciding on what size  pump to use you must know the approximate gallonage of the pond, the  actual lift of water (pumping head) and what you want to achieve with the water.

The actual lift of water is referred to as the ‘pumping head’. This is the vertical lift from the pond surface to the discharge point, as the head gets greater the pumping rate will be reduced. You need to be certain that the pump will give you the required rate at the required head. In order to maximise pumping rates pipe work should be kept as simple and as short as possible. 

For a waterfall allow about 50 gallons per inch of width (100 litres per cm of width). This will give you a thin continous sheet of water.

For a fountain you allow about :- 120 gallons (550  litres)  per hour for a simple rose jet.






  250 gallons (1100 litres) per hour for a bell or tulip jet.

For powering a filter most people turn over about half the pond per hour, however you must be sure that the filter is capable of coping with the flow.

Where a fountain and a waterfall are desired work out the gallonage for both and add them together to give the total required gallonage.

Remember a pump that is slightly to big can be controlled with a valve, but a pump that is to small can never be increased. 

It is important not to create strong currents in a pond, this can cause stress, tire the fish and cloud the water.

Some modern pumps are fitted with control boxes. They allow precise flow control with buttons and can make it possible to slow down the pump in winter, speed up during summer. When flow is turned down electric consumption will drop, enabling cost savings.

J.W.G. stock leading makes of submersible pump, Oase, Pond Tec and Evolution Aqua. These are all very reliable and carry 2 or 3 year guarantee. J.W.G. offers a great replace on the spot policy with returns. 

